# Daily Sowing Capacity (ha/day)

bar width (m) × speed (km/h) × working hours/day ÷ 10 = ha/day

e.g. an 18m bar at 10 km/h for 14 hours ≈ 18 × 10 × 14 ÷ 10 = 252 ha/day — then most growers knock 10–20% off that for turning, refilling, and breakdowns.

If you don't know your own number, here's where to find it:

- Your own records — most growers know roughly how many days it took to sow a known area last season; that's the most reliable figure, since it already bakes in real-world efficiency.

- Your agronomist or local Ag Bureau/grower group — SA has active regional groups (e.g. Yeruga, Hart Field Site Group, MacKillop Farm Management Group) who often discuss this kind of benchmark.

- Machinery dealer specs — bar width and rated speed for your actual seeder, which you can plug into the formula above.

- GRDC or PIRSA extension material — more geared to agronomy than machinery throughput, but worth a look for a citable reference.